CABLE & WIRE FOR ELECTRICAL, ELV AND INDUSTRIAL PROJECTS

# SPT-1 / SPT-2 / SPT-3 Lamp Cord

Exact model: SPT-1 | SPT-2 | SPT-3

PRODUCT IMAGES

| SPT-1 ¦ SPT-2 ¦ SPT-3 · Primary | SPT-1 ¦ SPT-2 ¦ SPT-3 · Gallery |
| --- | --- |

PRODUCT OVERVIEW

NexPro SPT-1 / SPT-2 / SPT-3 Lamp Cord is supplied as a configurable lamp cord product family. Catalog models are preserved exactly as SPT-1, SPT-2, SPT-3. Use the source-derived tables below to select the required conductor construction, core arrangement, cross-section and dimensions.

| Product line | Cable & Wire for Electrical, ELV and Industrial Projects |
| --- | --- |
| Series / family | Lamp cord |
| Exact model | SPT-1 ¦ SPT-2 ¦ SPT-3 |
| Applications | Equipment connections, Flexible wiring, OEM electrical assemblies |

MODEL IDENTIFICATION

| Exact model | Product / variant | Cable category | Catalog model | Configuration |
| --- | --- | --- | --- | --- |
| SPT-1 ¦ SPT-2 ¦ SPT-3 | SPT-1 / SPT-2 / SPT-3 Lamp Cord | Lamp cord | SPT-1 ¦ SPT-2 ¦ SPT-3 | Select from source table |

TECHNICAL SPECIFICATIONS

| Parameter | Specification |
| --- | --- |
| Cable category | Lamp cord |
| Catalog model | SPT-1 ¦ SPT-2 ¦ SPT-3 |
| Configuration | Select from source table |
| Manufacturer models | SPT-1 / SPT-2 / SPT-3 |
| SPT - 1 | Size (AWG): — ¦ No. of Conductors: — ¦ Conductor Stranding (No./AWG): — ¦ Insulation Thickness (inches): — ¦ Overall Diameter (inches): — ¦ Ampacity (amps): — ¦ Weight (lbs/kft): — |
| SPT1-18/2 | Size (AWG): 18 ¦ No. of Conductors: 2 ¦ Conductor Stranding (No./AWG): 41/34 ¦ Insulation Thickness (inches): 0.030 ¦ Overall Diameter (inches): 0.11 x 0.22 ¦ Ampacity (amps): 10 ¦ Weight (lbs/kft): 20 |
| SPT1-18/3 | Size (AWG): 18 ¦ No. of Conductors: 3 ¦ Conductor Stranding (No./AWG): 41/34 ¦ Insulation Thickness (inches): 0.030 ¦ Overall Diameter (inches): 0.11 x 0.33 ¦ Ampacity (amps): 10 ¦ Weight (lbs/kft): 30 |
| SPT – 2 | Size (AWG): — ¦ No. of Conductors: — ¦ Conductor Stranding (No./AWG): — ¦ Insulation Thickness (inches): — ¦ Overall Diameter (inches): — ¦ Ampacity (amps): — ¦ Weight (lbs/kft): — |
| SPT2-18/2 | Size (AWG): 18 ¦ No. of Conductors: 2 ¦ Conductor Stranding (No./AWG): 41/34 ¦ Insulation Thickness (inches): 0.045 ¦ Overall Diameter (inches): 0.14 x 0.27 ¦ Ampacity (amps): 10 ¦ Weight (lbs/kft): 27 |
| SPT2-16/2 | Size (AWG): 16 ¦ No. of Conductors: 2 ¦ Conductor Stranding (No./AWG): 65/34 ¦ Insulation Thickness (inches): 0.045 ¦ Overall Diameter (inches): 0.15 x 0.30 ¦ Ampacity (amps): 13 ¦ Weight (lbs/kft): 36 |
| SPT2-16/3 | Size (AWG): 16 ¦ No. of Conductors: 3 ¦ Conductor Stranding (No./AWG): 65/34 ¦ Insulation Thickness (inches): 0.045 ¦ Overall Diameter (inches): 0.15 x 0.45 ¦ Ampacity (amps): 10 ¦ Weight (lbs/kft): 54 |
| SPT – 3 | Size (AWG): — ¦ No. of Conductors: — ¦ Conductor Stranding (No./AWG): — ¦ Insulation Thickness (inches): — ¦ Overall Diameter (inches): — ¦ Ampacity (amps): — ¦ Weight (lbs/kft): — |

ENGINEERING TABLES

SPT-1 | SPT-2 | SPT-3 · Source Specification Table 1

| Part Number | Size (AWG) | No. of Conductors | Conductor Stranding (No./AWG) | Insulation Thickness (inches) | Overall Diameter (inches) | Ampacity (amps) | Weight (lbs/kft) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SPT - 1 | — | — | — | — | — | — | — |
| SPT1-18/2 | 18 | 2 | 41/34 | 0.030 | 0.11 x 0.22 | 10 | 20 |
| SPT1-18/3 | 18 | 3 | 41/34 | 0.030 | 0.11 x 0.33 | 10 | 30 |
| SPT – 2 | — | — | — | — | — | — | — |
| SPT2-18/2 | 18 | 2 | 41/34 | 0.045 | 0.14 x 0.27 | 10 | 27 |
| SPT2-16/2 | 16 | 2 | 65/34 | 0.045 | 0.15 x 0.30 | 13 | 36 |
| SPT2-16/3 | 16 | 3 | 65/34 | 0.045 | 0.15 x 0.45 | 10 | 54 |
| SPT – 3 | — | — | — | — | — | — | — |
| SPT3-16/2 | 16 | 2 | 65/34 | 0.060 | 0.18 x 0.36 | 13 | 47 |
| SPT3-14/2 | 14 | 2 | 41/30 | 0.060 | 0.20 x 0.40 | 18 | 61 |
| SPT3-14/3 | 14 | 3 | 41/30 | 0.060 | 0.20 x 0.60 | 15 | 92 |
| SPT3-12/2 | 12 | 2 | 65/30 | 0.060 | 0.22 x 0.44 | 25 | 82 |
| SPT3-12/3 | 12 | 3 | 65/30 | 0.060 | 0.22 x 0.66 | 20 | 123 |