CCTV & SURVEILLANCE SYSTEM

# NexPro 5MP Bullet (Triode Lamp, 24-LED, Audio)

Exact model: NexPro-BL1070M55-5MP-TRI

PRODUCT IMAGES

NexPro-BL1070M55-5MP-TRI · Primary

PRODUCT OVERVIEW

5MP full-color, triode lamp board, 24 LEDs (1W), built-in mic, 4000V surge protection

| Product line | CCTV & Surveillance System |
| --- | --- |
| Series / family | 1070 |
| Exact model | NexPro-BL1070M55-5MP-TRI |
| Applications | Parking Lot, Industrial, Large Perimeter |

MODEL IDENTIFICATION

| Exact model | Product / variant | Resolution | Lens | Illuminator |
| --- | --- | --- | --- | --- |
| NexPro-BL1070M55-5MP-TRI | NexPro 5MP Bullet (Triode Lamp, 24-LED, Audio) | 5MP | F1.4 5MP fixed | 24-LED triode (1W) |

TECHNICAL SPECIFICATIONS

| Parameter | Specification |
| --- | --- |
| Resolution | 5MP (2592×1944) @ real-time |
| Lens | F1.4 5MP fixed lens |
| Illuminator | 24-LED triode (1W); 24-LED dual light (12×20mil IR + 12×1W warm white) |
| Camera Type | 5MP Full-Color Network Bullet Camera with Triode Lamp Board |
| Solution | Tianview TB38M55 |
| Image Sensor | 1/2.7″ 5MP CMOS, low illumination |
| Lamp Board | Triode circuit design |
| Audio | Built-in microphone |
| Protection | Metal front cover, 4000V lightning surge protection |
| Light Control | 3 switchable modes |
| Quotation Type | Bulk project quotation |
| OEM/ODM | Available for selected orders |
| Documents | Datasheet and installation reference |
| Category | CCTV System |

APPLICATIONS & RFQ CONFIRMATION

| Typical applications | Confirm in the project RFQ |
| --- | --- |
| Parking Lot Industrial Large Perimeter | Confirm the exact model and required configuration. Confirm quantity, destination, packing, branding, and documentation requirements. |

Project supply: MOQ — Confirm by project quantity · Lead time — Confirm by project list · FOB port — China main ports · OEM/ODM — Available; confirm project scope

PROJECT CONFIGURATION CHECKLIST

| Exact model | NexPro-BL1070M55-5MP-TRI |
| --- | --- |
| Application and site conditions | Confirm installation environment and operating requirements. |
| Quantity and model mix | Confirm forecast quantity, phase plan and required variants. |
| Interfaces and accessories | Confirm compatible interfaces, accessories and system integration scope. |
| Finish, branding and packing | Confirm only when project customization is required. |
| Documentation and destination | Confirm drawings, datasheets, packing documents and destination country or port. |