CCTV & SURVEILLANCE SYSTEM

# 9013 Series

Models: NexPro-BL9013D3-4MP-TRI | NexPro-BL9013Y51-4MP-TRI | NexPro-BL9013M55-5MP-TRI

PRODUCT IMAGES

NexPro-BL9013D3-4MP-TRI · Primary

PRODUCT OVERVIEW

4MP full-color, triode lamp, 24 LEDs (1W), built-in mic, 4000V surge protection

| Product line | CCTV & Surveillance System |
| --- | --- |
| Series / family | 9013 |
| Exact model | NexPro-BL9013D3-4MP-TRI ¦ NexPro-BL9013Y51-4MP-TRI ¦ NexPro-BL9013M55-5MP-TRI |
| Applications | Harsh Environment, Critical Perimeter, Industrial |

MODEL COMPARISON

| Exact model | Product / variant | Resolution | Lens | Illuminator |
| --- | --- | --- | --- | --- |
| NexPro-BL9013D3-4MP-TRI | NexPro 4MP Premium Bullet (Triode, 24-LED, Audio) | 4MP | F1.4 5MP fixed | 24-LED triode (1W) |
| NexPro-BL9013Y51-4MP-TRI | NexPro 4MP Premium Bullet (5MP Sensor, Triode, 24-LED, Audio) | 4MP (5MP sensor) | F1.4 5MP fixed | 24-LED triode (1W) |
| NexPro-BL9013M55-5MP-TRI | NexPro 5MP Premium Bullet (Triode, 24-LED, Audio) | 5MP | F1.4 5MP fixed | 24-LED triode (1W) |

TECHNICAL SPECIFICATIONS

| Parameter | NexPro-BL9013D3-4MP-TRI | NexPro-BL9013Y51-4MP-TRI | NexPro-BL9013M55-5MP-TRI |
| --- | --- | --- | --- |
| Resolution | 4MP (2560×1440) @ real-time | 4MP (5MP sensor); 4MP (2560×1440) @ real-time (5MP sensor for superior low-light performance) | 5MP (2592×1944) @ real-time |
| Lens | F1.4 5MP fixed lens | F1.4 5MP fixed lens | F1.4 5MP fixed lens |
| Illuminator | 24-LED triode (1W); 24-LED dual light (12×20mil IR + 12×1W warm white) | 24-LED triode (1W); 24-LED dual light (12×20mil IR + 12×1W warm white) | 24-LED triode (1W); 24-LED dual light (12×20mil IR + 12×1W warm white) |
| Camera Type | 4MP Full-Color Network Bullet Camera (9013 Premium Housing) | 4MP Full-Color Network Bullet Camera (5MP Sensor, 9013 Housing) | 5MP Full-Color Network Bullet Camera (9013 Premium Housing) |
| Solution | Tianview TR38Y33 | Tianview TB38Y51 | Tianview TB38M55 |
| Image Sensor | 1/2.7″ 4MP CMOS, low illumination | 1/2.7″ 5MP CMOS, low illumination | 1/2.7″ 5MP CMOS, low illumination |
| Lamp Board | Triode circuit design | Triode circuit design | Triode circuit design |
| Audio | Built-in microphone | Built-in microphone | Built-in microphone |
| Protection | Metal front cover, 4000V lightning surge protection | Metal front cover, 4000V lightning surge protection | Metal front cover, 4000V lightning surge protection |
| Light Control | 3 switchable modes | 3 switchable modes | 3 switchable modes |
| Quotation Type | Bulk project quotation | Bulk project quotation | Bulk project quotation |
| OEM/ODM | Available for selected orders | Available for selected orders | Available for selected orders |
| Documents | Datasheet and installation reference | Datasheet and installation reference | Datasheet and installation reference |
| Category | CCTV System | CCTV System | CCTV System |

APPLICATIONS & RFQ CONFIRMATION

| Typical applications | Confirm in the project RFQ |
| --- | --- |
| Harsh Environment Critical Perimeter Industrial Logistics Hub Mission-Critical Perimeter Industrial Campus Oil & Gas | Confirm the exact model and required configuration. Confirm quantity, destination, packing, branding, and documentation requirements. |

Project supply: MOQ — Confirm by project quantity · Lead time — Confirm by project list · FOB port — China main ports · OEM/ODM — Available; confirm project scope

PROJECT CONFIGURATION CHECKLIST

| Exact model | NexPro-BL9013D3-4MP-TRI |
| --- | --- |
| Application and site conditions | Confirm installation environment and operating requirements. |
| Quantity and model mix | Confirm forecast quantity, phase plan and required variants. |
| Interfaces and accessories | Confirm compatible interfaces, accessories and system integration scope. |
| Finish, branding and packing | Confirm only when project customization is required. |
| Documentation and destination | Confirm drawings, datasheets, packing documents and destination country or port. |