RACK KVM & REMOTE CONSOLE MANAGEMENT

# SP8732 32-Port CAT5 Network KVM

Exact model: SP8732

PRODUCT IMAGES

| SP8732 · Primary | SP8732 · Diagram · Series Reference View |
| --- | --- |

PRODUCT OVERVIEW

CAT5/IP KVM consoles for high-density server management and cascading.

| Product line | Rack KVM & Remote Console Management |
| --- | --- |
| Series / family | kvm-cat5 |
| Exact model | SP8732 |
| Applications | High-density server racks, CAT5 KVM extension, Data center operations |

MODEL IDENTIFICATION

| Exact model | Product / variant | Port Connection | Port Count | Input / Output |
| --- | --- | --- | --- | --- |
| SP8732 | SP8732 32-Port CAT5 Network KVM | RJ45 | 32 | PS/2 / USB dual-interface converter module |

TECHNICAL SPECIFICATIONS

| Parameter | Specification |
| --- | --- |
| Port Connection | RJ45 |
| Port Count | 32 |
| Input / Output | PS/2 / USB dual-interface converter module |
| Resolution | 1280 × 1024 |
| Scan Interval | 5–255 seconds |
| Supported Systems | Windows, Vista, Linux, etc. |
| Power Input | A C100-240V, 50-60Hz, 1 A |
| Power Consumption | 120V at 23 W; 230V at 25 W |
| Optional Modules | CAT5 module or IP remote-access card module |
| Host Interface | USB / PS/2 |
| Cascading | Supports bus and tree topologies |
| Dimensions | 485 × 570 × 44 mm |
| Warranty | 3 years for main unit; 1 year for display |
| Product series | 32-Port CAT5/IP KVM Series |
| Product category | KVM |

APPLICATIONS & RFQ CONFIRMATION

| Typical applications | Confirm in the project RFQ |
| --- | --- |
| High-density server racks CAT5 KVM extension Data center operations | Confirm the application, rack or room position, destination-market interface and mounting requirement. Confirm electrical, network, accessory or protection scope from the product family and project drawings. Confirm quantity, labeling, packing and required documents before purchase order. |

Project supply: MOQ — Confirm by model and project quantity · Lead time — Confirm after configuration review · FOB port — Confirm in project quotation · OEM/ODM — To be confirmed

PROJECT CONFIGURATION CHECKLIST

| Exact model | SP8732 |
| --- | --- |
| Application and site conditions | Confirm installation environment and operating requirements. |
| Quantity and model mix | Confirm forecast quantity, phase plan and required variants. |
| Interfaces and accessories | Confirm compatible interfaces, accessories and system integration scope. |
| Finish, branding and packing | Confirm only when project customization is required. |
| Documentation and destination | Confirm drawings, datasheets, packing documents and destination country or port. |