RACK PDU & POWER DISTRIBUTION

# Circuit Breaker & Overload Protection PDU Series

Models: DT7188 | SP5071 | SP51103 | SP5188 | SP6081

PRODUCT IMAGES

| DT7188 · Primary | DT7188 · Gallery | SP5071 · Primary | SP5071 · Gallery |
| --- | --- | --- | --- |

| SP5071 · Gallery | SP5071 · Gallery | SP51103 · Primary | SP51103 · Gallery |
| --- | --- | --- | --- |

PRODUCT OVERVIEW

Rack power distribution with circuit-breaker or overload-protection modules for enhanced overcurrent protection.

| Product line | Rack PDU & Power Distribution |
| --- | --- |
| Series / family | pdu-breaker |
| Exact model | DT7188 ¦ SP5071 ¦ SP51103 ¦ SP5188 ¦ SP6081 |
| Applications | Server and network racks, Equipment cabinets, Overload-protected distribution |

MODEL COMPARISON

| Exact model | Product / variant | Input Current | Input Cable | Input Plug |
| --- | --- | --- | --- | --- |
| DT7188 | DT7188 10 A Switched Overload-Protected PDU | 10 A | 3 m, 1.5 mm² | 10A Chinese-standard plug |
| SP5071 | SP5071 Circuit Breaker & Overload Protection PDU | 16 A | 3 m, 2.5 mm² | Not provided |
| SP51103 | SP51103 10-Outlet 3-in-1 Protection PDU | 16 A | 3 m, 2.5 mm² | 16A Chinese-standard plug |
| SP5188 | SP5188 16 A Switched Overload-Protected PDU | 16 A | 3 m, 2.5 mm² | 16A Chinese-standard plug |
| SP6081 | SP6081 32 A High-Power Circuit-Breaker PDU | 32 A | Not provided | Not provided |

TECHNICAL SPECIFICATIONS

Models: DT7188 | SP5071 | SP51103 | SP5188

| Parameter | DT7188 | SP5071 | SP51103 | SP5188 |
| --- | --- | --- | --- | --- |
| Input Current | 10 A | 16 A | 16 A | 16 A |
| Input Cable | 3 m, 1.5 mm² | 3 m, 2.5 mm² | 3 m, 2.5 mm² | 3 m, 2.5 mm² |
| Input Plug — Key Specifications | 10A Chinese-standard plug | Not provided | 16A Chinese-standard plug | 16A Chinese-standard plug |
| Function Module | Switch + overload protection | Circuit breaker | 3-in-1 protection | Switch + overload protection |
| Output | 8 × 10 A universal outlets | 7 × 16 A Chinese-standard outlets | 10 × 10 A universal outlets | 8 × 10 A universal outlets |
| Rated Load | 2500 W | 4000 W | 4000 W | 4000 W |
| Dimensions | 485 × 44.5 × 44.5 mm | 485 × 44.5 × 44.5 mm | 485 × 44.5 × 44.5 mm | 485 × 44.5 × 44.5 mm |
| Product series | Circuit Breaker & Overload Protection PDU Series | Circuit Breaker & Overload Protection PDU Series | Circuit Breaker & Overload Protection PDU Series | Circuit Breaker & Overload Protection PDU Series |
| Product category | PDU | Not provided | PDU | PDU |
| Configurations | Not provided | 2 | Not provided | Not provided |
| Input Plug — Source Technical Data | Not provided | 16A Chinese-standard plug | Not provided | Not provided |
| Entity type | Not provided | Product group with selectable configurations | Not provided | Not provided |

Models: SP6081

| Parameter | Specification |
| --- | --- |
| Cable Specification | 3 × 4 mm² |
| Input Current | 32 A |
| Rated Load | 6000 W |
| Input Plug | 32A industrial plug |
| Function Module | Circuit breaker |
| Output | 8 × 16 A Chinese-standard outlets |
| Dimensions | 485 × 44.5 × 44.5 mm |
| Product series | Circuit Breaker & Overload Protection PDU Series |
| Product category | PDU |

APPLICATIONS & RFQ CONFIRMATION

| Typical applications | Confirm in the project RFQ |
| --- | --- |
| Server and network racks Equipment cabinets Overload-protected distribution | Confirm the application, rack or room position, destination-market interface and mounting requirement. Confirm electrical, network, accessory or protection scope from the product family and project drawings. Confirm quantity, labeling, packing and required documents before purchase order. |

Project supply: MOQ — Confirm by model and project quantity · Lead time — Confirm after configuration review · FOB port — Confirm in project quotation · OEM/ODM — To be confirmed

PROJECT CONFIGURATION CHECKLIST

| Exact model | DT7188 |
| --- | --- |
| Application and site conditions | Confirm installation environment and operating requirements. |
| Quantity and model mix | Confirm forecast quantity, phase plan and required variants. |
| Interfaces and accessories | Confirm compatible interfaces, accessories and system integration scope. |
| Finish, branding and packing | Confirm only when project customization is required. |
| Documentation and destination | Confirm drawings, datasheets, packing documents and destination country or port. |