RACK CABINETS & DATA CENTER CONTAINMENT

# K5-61242 42U 1200 mm-Deep K5 Server Cabinet

Exact model: K5-61242

PRODUCT IMAGES

| K5-61242 · Primary | K5-61242 · Diagram · Series Reference View |
| --- | --- |

PRODUCT OVERVIEW

Heavy-duty 19-inch server cabinets for higher load and deeper equipment configurations.

| Product line | Rack Cabinets & Data Center Containment |
| --- | --- |
| Series / family | k5 |
| Exact model | K5-61242 |
| Applications | Server rooms, Data-room racks, High-load equipment cabinets |

MODEL IDENTIFICATION

| Exact model | Product / variant | Type | Dimensions | Height |
| --- | --- | --- | --- | --- |
| K5-61242 | K5-61242 42U 1200 mm-Deep K5 Server Cabinet | Server cabinet | 600 × 1200 × 2000 mm | 42U |

TECHNICAL SPECIFICATIONS

| Parameter | Specification |
| --- | --- |
| Type | Server cabinet |
| Dimensions | 600 × 1200 × 2000 mm |
| Height | 42U |
| Mounting Rail Spacing | 485 mm (19-inch standard) |
| Sheet Thickness | mounting rails 2.0 mm; other panels 1.2 mm |
| Material / Finish | SPCC cold-rolled steel, degreased and powder-coated |
| Series Load Capacity | K5 series catalog load rating on page 13: ≤700 kg |
| Standard Configuration | PDU × 1、cage nuts and screws × 20、M12leveling feet × 4、42Umounting rails × 4、front and rear mesh doors × 1、side panels × 2、fans × 4、shelves × 3、casters × 4 |
| Recommended Equipment Depth | <1050 mm |
| Product series | K5 Heavy-Duty Server Cabinet Series |
| Product category | Data Center Cabinet |

APPLICATIONS & RFQ CONFIRMATION

| Typical applications | Confirm in the project RFQ |
| --- | --- |
| Server rooms Data-room racks High-load equipment cabinets | Confirm the application, rack or room position, destination-market interface and mounting requirement. Confirm electrical, network, accessory or protection scope from the product family and project drawings. Confirm quantity, labeling, packing and required documents before purchase order. |

Project supply: MOQ — Confirm by model and project quantity · Lead time — Confirm after configuration review · FOB port — Confirm in project quotation · OEM/ODM — To be confirmed

PROJECT CONFIGURATION CHECKLIST

| Exact model | K5-61242 |
| --- | --- |
| Application and site conditions | Confirm installation environment and operating requirements. |
| Quantity and model mix | Confirm forecast quantity, phase plan and required variants. |
| Interfaces and accessories | Confirm compatible interfaces, accessories and system integration scope. |
| Finish, branding and packing | Confirm only when project customization is required. |
| Documentation and destination | Confirm drawings, datasheets, packing documents and destination country or port. |