RACK CABINETS & DATA CENTER CONTAINMENT

# K1 Network & Server Cabinet Series

Models: K1-5006 | K1-5012 | K1-6042 | K1-6638 | K1-6642

PRODUCT IMAGES

| K1-5006 · Primary | K1-5006 · Gallery | K1-5006 · Diagram · Series Reference View | K1-5012 · Primary |
| --- | --- | --- | --- |

| K1-5012 · Gallery | K1-5012 · Diagram · Series Reference View | K1-6042 · Primary | K1-6042 · Gallery |
| --- | --- | --- | --- |

PRODUCT OVERVIEW

19-inch network and server cabinets for standard IT, networking and security equipment.

| Product line | Rack Cabinets & Data Center Containment |
| --- | --- |
| Series / family | k1 |
| Exact model | K1-5006 ¦ K1-5012 ¦ K1-6042 ¦ K1-6638 ¦ K1-6642 |
| Applications | Wall-mount network cabinets, CCTV and ELV equipment racks, Small server rooms |

MODEL COMPARISON

| Exact model | Product / variant | Type | Dimensions | Height |
| --- | --- | --- | --- | --- |
| K1-5006 | K1-5006 6U wall-mount network cabinet | Network cabinet / wall-mount cabinet | 350 × 550 × 400 mm | 6U |
| K1-5012 | K1-5012 12U Compact Network Cabinet | Network cabinet | 450 × 550 × 700 mm | 12U |
| K1-6042 | K1-6042 42U K1 server cabinet | Server cabinet | 600 × 1000 × 2000 mm | 42U |
| K1-6638 | K1-6638 38U K1 network cabinet | Network cabinet | 600 × 600 × 1800 mm | 38U |
| K1-6642 | K1-6642 42U K1 Cabinet (Duplicate Catalog Specification) | Catalog classification: server cabinet | catalog states 600 × 1000 × 2000 mm | Not provided |

TECHNICAL SPECIFICATIONS

Models: K1-5006 | K1-5012 | K1-6042 | K1-6638

| Parameter | K1-5006 | K1-5012 | K1-6042 | K1-6638 |
| --- | --- | --- | --- | --- |
| Type | Network cabinet / wall-mount cabinet | Network cabinet | Server cabinet | Network cabinet |
| Dimensions | 350 × 550 × 400 mm | 450 × 550 × 700 mm | 600 × 1000 × 2000 mm | 600 × 600 × 1800 mm |
| Height | 6U | 12U | 42U | 38U |
| Mounting Rail Spacing | 485 mm (19-inch standard) | 485 mm (19-inch standard) | 485 mm (19-inch standard) | 485 mm (19-inch standard) |
| Sheet Thickness | mounting rails 2.0 mm; other panels 1.0 mm | mounting rails 2.0 mm; other panels 1.0 mm | mounting rails 2.0 mm; other / main load-bearing beam approx. 1.1±0.1 mm | mounting rails 2.0 mm; other panels 1.1 mm |
| Material / Finish | SPCC cold-rolled steel, degreased and powder-coated | SPCC cold-rolled steel, degreased and powder-coated | SPCC cold-rolled steel, degreased and powder-coated | SPCC cold-rolled steel, degreased and powder-coated |
| Series Load Capacity | Wall-mount small-cabinet load rating: ≤50 kg | Small-cabinet catalog load rating: ≤50 kg | K1 series catalog load rating: ≤600 kg | ≤600kg |
| Standard Configuration | cage nuts and screws × 20、6Umounting rails × 4、glass door × 1、side panels × 2 | cage nuts and screws × 20、12Umounting rails × 4、glass door × 1、side panels × 2、shelves × 1、casters × 4 | PDU × 1、cage nuts and screws × 20、M12leveling feet × 4、42Umounting rails × 4、front and rear mesh doors × 1、side panels × 2、fans × 2、shelves × 3、casters × 4 | PDU × 1、cage nuts and screws × 20、M12leveling feet × 4、38Umounting rails × 4、front and rear mesh doors × 1、side panels × 2、fans × 1、shelves × 3、casters × 4 |
| Recommended Equipment Depth | <300 mm | <350 mm | <850 mm | <450 mm |
| Product series | K1 Network & Server Cabinet Series | K1 Network & Server Cabinet Series | K1 Network & Server Cabinet Series | K1 Network & Server Cabinet Series |
| Product category | Data Center Cabinet | Data Center Cabinet | Data Center Cabinet | Data Center Cabinet |

Models: K1-6642

| Parameter | Specification |
| --- | --- |
| Type | Catalog classification: server cabinet |
| Dimensions | catalog states 600 × 1000 × 2000 mm |
| Mounting Rail Spacing | 485 mm (19-inch standard) |
| Sheet Thickness | mounting rails 2.0 mm; main load-bearing beam approx. 1.1±0.1 mm |
| Material / Finish | SPCC cold-rolled steel, degreased and powder-coated |
| Standard Configuration | Same catalog specification as K1-6042 |
| Recommended Equipment Depth | <850 mm |
| Product series | K1 Network & Server Cabinet Series |
| Product category | Data Center Cabinet |

APPLICATIONS & RFQ CONFIRMATION

| Typical applications | Confirm in the project RFQ |
| --- | --- |
| Wall-mount network cabinets CCTV and ELV equipment racks Small server rooms | Confirm the application, rack or room position, destination-market interface and mounting requirement. Confirm electrical, network, accessory or protection scope from the product family and project drawings. Confirm quantity, labeling, packing and required documents before purchase order. |

Project supply: MOQ — Confirm by model and project quantity · Lead time — Confirm after configuration review · FOB port — Confirm in project quotation · OEM/ODM — To be confirmed

PROJECT CONFIGURATION CHECKLIST

| Exact model | K1-5006 |
| --- | --- |
| Application and site conditions | Confirm installation environment and operating requirements. |
| Quantity and model mix | Confirm forecast quantity, phase plan and required variants. |
| Interfaces and accessories | Confirm compatible interfaces, accessories and system integration scope. |
| Finish, branding and packing | Confirm only when project customization is required. |
| Documentation and destination | Confirm drawings, datasheets, packing documents and destination country or port. |