UPS UNINTERRUPTIBLE POWER SUPPLY

# DC Mini UPS 45W-120W Series

Exact model: NX-UP-DC-002

PRODUCT IMAGES

| NX-UP-DC-002 · Primary | NX-UP-DC-002 · Gallery |
| --- | --- |

PRODUCT OVERVIEW

Higher-power DC Mini UPS series with POE support for CCTV cameras, network equipment, and multi-voltage devices. Four model variants from 45W to 120W.

| Product line | UPS Uninterruptible Power Supply |
| --- | --- |
| Series / family | Standalone model |
| Exact model | NX-UP-DC-002 |
| Applications | CCTV systems, Routers and network terminals, POE-powered devices, Multi-voltage small equipment |

MODEL IDENTIFICATION

| Exact model | Product / variant | Topology | Power Range | Phase |
| --- | --- | --- | --- | --- |
| NX-UP-DC-002 | DC Mini UPS 45W-120W Series | DC UPS with Li-ion battery & POE | 45W-120W | 100–240VAC, 50–60Hz input / DC / USB / PoE options output |

TECHNICAL SPECIFICATIONS

| Parameter | Specification |
| --- | --- |
| Topology | DC UPS with Li-ion battery & POE |
| Power Range | 45W-120W |
| Phase | 100–240VAC, 50–60Hz input / DC / USB / PoE options output |
| Form Factor | Compact DC Unit |
| Waveform | DC output |
| Power Factor | N/A (DC output) |
| Battery Types | Lithium-ion / LiFePO4 |
| SKU | NX-UP-DC-002 |
| Available Configurations | DCP-045W; DCP-60W; DCP-100W; DCP-120W |
| Exact Configuration | Confirm one listed configuration during RFQ review |
| Destination Requirements | To be confirmed |

ENGINEERING TABLES

NX-UP-DC-002 · Complete Model Specifications — DCP-045W | DCP-60W | DCP-100W | DCP-120W

| Parameter | DCP-045W | DCP-60W | DCP-100W | DCP-120W |
| --- | --- | --- | --- | --- |
| Rated capacity | 45W | 60W | 100W | 120W |
| Battery configuration | 4 / 8 cells; 18650 | 4 / 8 cells; 18650 / 32650 | 4 / 8 cells; 18650 / 32650 | 4 / 8 cells; 18650 / 32650 |
| Dimensions | 180×130×64mm | 214×158.5×86mm | 214×158.5×86mm | 214×158.5×86mm |
| Net weight | 0.83kg | 1.85kg | 1.88kg | 1.9kg |
| Transfer time | Not provided | Not provided | Not provided | Not provided |
| Efficiency | AC >85%; Battery >90% | AC >85%; Battery >90% | AC >85%; Battery >90% | AC >85%; Battery >90% |
| Model configuration — Rated capacity | 45W | 60W | 100W | 120W |
| Model configuration — Battery configuration | 4 / 8 cells; 18650 | 4 / 8 cells; 18650 / 32650 | 4 / 8 cells; 18650 / 32650 | 4 / 8 cells; 18650 / 32650 |
| Model configuration — Product dimensions | 180×130×64mm | 214×158.5×86mm | 214×158.5×86mm | 214×158.5×86mm |
| Model configuration — Net weight | 0.83kg | 1.85kg | 1.88kg | 1.9kg |
| Model configuration — Efficiency | AC >85%; Battery >90% | AC >85%; Battery >90% | AC >85%; Battery >90% | AC >85%; Battery >90% |
| Input — Input voltage | 100–240VAC | 100–240VAC | 100–240VAC | 100–240VAC |
| Input — Input frequency | 50–60Hz | 50–60Hz | 50–60Hz | 50–60Hz |
| Battery — Battery chemistry | Lithium-ion / LiFePO4 | Lithium-ion / LiFePO4 | Lithium-ion / LiFePO4 | Lithium-ion / LiFePO4 |
| Battery — Battery quantity | 4 / 8 | 4 / 8 | 4 / 8 | 4 / 8 |
| Battery — Battery cell size | 18650 | 18650 / 32650 | 18650 / 32650 | 18650 / 32650 |
| Battery — Cell capacity | 2000–2600mAh | 2000–6000mAh | 2000–6000mAh | 2000–6000mAh |
| Protection — Protection function | Over-charge, over-discharge, over-current, overload, over-temperature and short circuit | Over-charge, over-discharge, over-current, overload, over-temperature and short circuit | Over-charge, over-discharge, over-current, overload, over-temperature and short circuit | Over-charge, over-discharge, over-current, overload, over-temperature and short circuit |
| Environment — Operating temperature | 0–40°C | 0–40°C | 0–40°C | 0–40°C |
| Environment — Storage temperature | -10–70°C | -10–70°C | -10–70°C | -10–70°C |
| Environment — Relative humidity | 10–90% RH | 10–90% RH | 10–90% RH | 10–90% RH |
| Environment — Altitude | <3000m | <3000m | <3000m | <3000m |
| Environment — Noise level | <30dB | <30dB | <30dB | <30dB |
| Housing — Housing material | ABS | ABS | ABS | ABS |
| Housing — Housing color | Black / White | Black / White | Black / White | Black / White |
| Output — USB output | 5V/2.4A | 5V/2.4A | 5V/2.4A | 5V/2.4A |
| Output — DC 5V output | 5V/2.4A | 5V/2.4A | 5V/2.4A | 5V/2.4A |
| Output — DC 9V / 12V output | 9V(12V)/2A | 9V(12V)/2A | 9V(12V)/2A | 9V(12V)/2A |
| Output — DC 12V output | 12V/3.5A | 12V/5A | 12V/8A | 12V/10A |
| Output — PoE 15V output | 15V/1.5A | Not provided | 15V/1.5A | 15V/1.5A |
| Output — PoE 24V output | 24V/1A | 24V/1A | 24V/1A | 24V/1A |
| Output — PoE 48V output | 48V/0.6A | 48V/0.6A (optional) | 48V/0.6A (optional) | 48V/0.6A (optional) |
| Physical — Package dimensions | 195×180×70mm | 230×230×95mm | 230×230×95mm | 230×230×95mm |
| Physical — Gross weight | 1.2kg | 2.6kg | 2.63kg | 2.65kg |
| Output — DC 19V output | Not provided | 19V/3A | 19V/5A | 19V/6A |