UPS UNINTERRUPTIBLE POWER SUPPLY

# Standby UPS 650VA-3000VA Series

Exact model: NX-UP-ST-001

PRODUCT IMAGES

| NX-UP-ST-001 · Primary | NX-UP-ST-001 · Gallery |
| --- | --- |

PRODUCT OVERVIEW

Cost-effective offline UPS series providing reliable battery backup and surge protection for personal computers, POS systems, and small office equipment. Available in 650VA to 3000VA capacities.

| Product line | UPS Uninterruptible Power Supply |
| --- | --- |
| Series / family | Standalone model |
| Exact model | NX-UP-ST-001 |
| Applications | Personal computers, Printers, POS terminals, Security systems, Fax machines, Modems and routers |

MODEL IDENTIFICATION

| Exact model | Product / variant | Topology | Power Range | Phase |
| --- | --- | --- | --- | --- |
| NX-UP-ST-001 | Standby UPS 650VA-3000VA Series | Offline (Standby) | 650VA-3000VA / 390W-1800W | Single phase input / Single phase output |

TECHNICAL SPECIFICATIONS

| Parameter | Specification |
| --- | --- |
| Topology | Offline (Standby) |
| Power Range | 650VA-3000VA / 390W-1800W |
| Phase | Single phase input / Single phase output |
| Form Factor | Tower |
| Waveform | Simulated sine wave |
| Power Factor | 0.6 |
| Battery Types | VRLA sealed lead-acid |
| SKU | NX-UP-ST-001 |
| Available Configurations | SP650; SP1000; SP1500; SP2000; SP3000 |
| Exact Configuration | Confirm one listed configuration during RFQ review |
| Destination Requirements | To be confirmed |

ENGINEERING TABLES

NX-UP-ST-001 · Complete Model Specifications — SP650 | SP1000 | SP1500 | SP2000

| Parameter | SP650 | SP1000 | SP1500 | SP2000 |
| --- | --- | --- | --- | --- |
| Rated capacity | 650VA / 390W | 1000VA / 600W | 1500VA / 900W | 2000VA / 1200W |
| Battery configuration | 12V/7AH × 1 | 12V/9AH × 1 | 12V/9AH × 2 | 12V/9AH × 2 |
| Dimensions | 320×90×140mm | 320×90×140mm | 350×115×275mm | 350×115×275mm |
| Net weight | 5.1kg | 5.7kg | 12.2kg | 12.35kg |
| Transfer time | ≤6ms | ≤6ms | ≤6ms | ≤6ms |
| Efficiency | Not provided | Not provided | Not provided | Not provided |
| Model configuration — Rated capacity | 650VA / 390W | 1000VA / 600W | 1500VA / 900W | 2000VA / 1200W |
| Model configuration — Battery configuration | 12V/7AH × 1 | 12V/9AH × 1 | 12V/9AH × 2 | 12V/9AH × 2 |
| Model configuration — Product dimensions | 320×90×140mm | 320×90×140mm | 350×115×275mm | 350×115×275mm |
| Model configuration — Net weight | 5.1kg | 5.7kg | 12.2kg | 12.35kg |
| Model configuration — Transfer time | ≤6ms | ≤6ms | ≤6ms | ≤6ms |
| Input — Input voltage range | 140–280VAC | 140–280VAC | 140–280VAC | 140–280VAC |
| Input — Input frequency | 50Hz; tolerance requires confirmation | 50Hz; tolerance requires confirmation | 50Hz; tolerance requires confirmation | 50Hz; tolerance requires confirmation |
| Output — Voltage regulation | ±10% | ±10% | ±10% | ±10% |
| Output — Output frequency | 50Hz; tolerance requires confirmation | 50Hz; tolerance requires confirmation | 50Hz; tolerance requires confirmation | 50Hz; tolerance requires confirmation |
| Charging — Charging time | 10–16h to 90% capacity | 10–16h to 90% capacity | 10–16h to 90% capacity | 10–16h to 90% capacity |
| Protection — Protection function | Discharge, short circuit and overload | Discharge, short circuit and overload | Discharge, short circuit and overload | Discharge, short circuit and overload |
| Display & Alarms — Display | LCD | LCD | LCD | LCD |
| Display & Alarms — Battery mode alarm | Every 4 seconds | Every 4 seconds | Every 4 seconds | Every 4 seconds |
| Display & Alarms — Low battery alarm | Every second | Every second | Every second | Every second |
| Display & Alarms — Overload alarm | Every second | Every second | Every second | Every second |
| Display & Alarms — Fault alarm | Continuous | Continuous | Continuous | Continuous |
| Environment — Operating temperature | 0–40°C | 0–40°C | 0–40°C | 0–40°C |
| Environment — Relative humidity | 0–90% RH | 0–90% RH | 0–90% RH | 0–90% RH |
| Environment — Noise level | <40dB | <40dB | <40dB | <40dB |

NX-UP-ST-001 · Complete Model Specifications — SP3000

| Parameter | SP3000 |
| --- | --- |
| Rated capacity | 3000VA / 1800W |
| Battery configuration | 12V/12AH × 2 |
| Dimensions | 350×115×275mm |
| Net weight | 15.8kg |
| Transfer time | ≤6ms |
| Efficiency | Not provided |
| Model configuration — Rated capacity | 3000VA / 1800W |
| Model configuration — Battery configuration | 12V/12AH × 2 |
| Model configuration — Product dimensions | 350×115×275mm |
| Model configuration — Net weight | 15.8kg |
| Model configuration — Transfer time | ≤6ms |
| Input — Input voltage range | 140–280VAC |
| Input — Input frequency | 50Hz; tolerance requires confirmation |
| Output — Voltage regulation | ±10% |
| Output — Output frequency | 50Hz; tolerance requires confirmation |
| Charging — Charging time | 10–16h to 90% capacity |
| Protection — Protection function | Discharge, short circuit and overload |
| Display & Alarms — Display | LCD |
| Display & Alarms — Battery mode alarm | Every 4 seconds |
| Display & Alarms — Low battery alarm | Every second |
| Display & Alarms — Overload alarm | Every second |
| Display & Alarms — Fault alarm | Continuous |
| Environment — Operating temperature | 0–40°C |
| Environment — Relative humidity | 0–90% RH |
| Environment — Noise level | <40dB |