NEXPRO

NexPro SW-POE1618G
18-Port Gigabit PoE Switch (16-Port PoE + 2 Gigabit Uplink)

Technical Specification Sheet | Document Version 1.0 | May 2026

Product Overview

The NexPro SW-POE1618G is an 18-port Gigabit PoE switch engineered specifically for medium to large IP surveillance and wireless access point installations. With 16 PoE-enabled ports delivering both data and DC power over a single Cat5e/Cat6 cable, and 2 Gigabit uplink ports for high-speed NVR or core switch connectivity, it eliminates the need for separate power cabling at every camera location. A robust 250W total power budget and 7.2 Gbps backplane bandwidth ensure every connected camera records without interruption.

Core Value Proposition
Extended Reach: Extend mode delivers power + data up to 250m at 10Mbps.
Gigabit Uplink: Two 1000Mbps uplink ports ensure zero bottleneck when aggregating 16 camera streams.
True Plug-and-Play: Zero configuration — auto-sensing PoE, instant device recognition.
Intelligent Management: VLAN isolation and per-port speed control via a single physical switch.
Surge-Protected: Built-in lightning protection, engineered for surveillance-grade 24/7 operation.

Key Features

- 16× 100Mbps PoE Ports — Simultaneous data and DC power delivery via standard Cat5e/Cat6 cable; auto-sensing IEEE 802.3af/at compliant powered devices. 250W total power budget supports fully loaded 16-channel camera deployments.

- 2× Gigabit Uplink Ports — Dedicated 1000Mbps uplink for NVR connection or cascading to core switch; no data bottleneck when aggregating all 16 camera streams simultaneously.

- Extend Mode (250m) — When enabled, ports 1–16 operate at forced 10Mbps with VLAN isolation, extending transmission distance to 250 metres — ideal for far-end perimeter cameras, remote gate installations, and parking lot coverage without additional repeaters or fibre converters.

- VLAN Isolation — Paired with Extend mode, isolates each PoE port from one another to prevent broadcast storms and enhance network security for dedicated surveillance traffic across large port counts.

- Wide Voltage PoE — DC 48–55V power supply, 250W total budget; supports both IEEE 802.3af (15.4W) and IEEE 802.3at (30W) powered devices simultaneously across all 16 ports.

- Plug-and-Play — No software configuration required; connect cameras and NVR, power on, and the system is operational within seconds.

- Compact Steel Housing — 312×185×46mm form factor fits standard 19-inch rack shelves and wall-mount enclosures.

- Surge Protection — Built-in lightning and surge protection for stable operation in outdoor cabinet and plant-floor installations.

- Broad Compatibility — Suitable for IP cameras, wireless APs, and other PoE-powered devices in retail, residential, school, hospital, campus, hotel, and SME environments.

Technical Specifications

| Product Name | 18-Port Gigabit Standard PoE Switch |
| --- | --- |
| Model Number | NexPro SW-POE1618G / 16F20GB |
| Forwarding Mode | Store-and-Forward |
| Transmission Distance | Standard 100m / Extend mode up to 250m |
| Backplane Bandwidth | 7.2 Gbps full-duplex |
| PoE Ports | Ports 1–16 (Power + Data, Extend mode supported) |
| Uplink Ports | 2× 1000Mbps RJ45 Gigabit (non-PoE) |
| Data Pin Assignment | 1, 2, 3, 6 |
| Power Pin Assignment | Pins 1-2 positive (+) / Pins 3-6 negative (-) |
| Voltage Specification | DC 48–55V |
| Standard Power Supply | DC 52V / 250W |
| PoE Protocols | IEEE 802.3at (PoE+), IEEE 802.3af (PoE) |
| Network Medium | Cat5 / Cat5e UTP (unshielded twisted pair) |
| Dimensions | 312 × 185 × 46 mm (L×W×H) |
| Extend/VLAN Switch | ON: Ports 1-16 forced 10Mbps / 250m reach / VLAN enabled OFF: Ports 1-16 at 100Mbps / 100m reach / VLAN disabled |

Extend & VLAN Mode Configuration

The physical slide switch on the unit provides instant operational mode switching — no software, no reboot loops, no complexity.

| Switch Position | Port Speed | Max Distance | VLAN Isolation | Recommended Use Case |
| --- | --- | --- | --- | --- |
| ON | 10 Mbps (forced) | 250 metres | Enabled | Perimeter fence cameras, remote gate cameras, standalone far-end installations where high frame rate is not critical but distance exceeds 100m |
| OFF (default) | 100 Mbps | 100 metres | Disabled | Standard indoor and near-field installations where full frame rate and resolution are required from every camera |

Deployment Scenarios

For Hotel Projects

- One SW-POE1618G per floor or per zone covers up to 16 corridor cameras (NexPro-BL series) and ceiling APs (NexPro-WiFi series) with a single switch.

- Gigabit uplink ports connect directly to the hotel's core network without creating a bottleneck, even with all 16 cameras recording simultaneously at 4Mbps.

- Extend mode covers parking lot far-end cameras without additional power cabling or fibre converters.

- VLAN isolation prevents a single faulty camera from flooding the entire floor's surveillance network.

For Industrial & Factory Sites

- 250W power budget comfortably supports 16 outdoor bullet cameras with IR illumination (NexPro-BL series), including heater-equipped models in cold storage areas.

- 7.2 Gbps backplane bandwidth ensures zero packet loss during continuous NVR recording across all channels.

- Metal housing and built-in surge protection withstand vibration, temperature fluctuations, and electrical noise common in plant environments.

- Gigabit uplink enables smooth integration with SCADA or MES monitoring overlays without bandwidth contention.

For System Integrators

- Auto-sensing PoE eliminates the need to manually calculate power budgets per port — the switch negotiates with each connected camera (NexPro-BL, NexPro-PT, or third-party ONVIF cameras).

- Two Gigabit uplink ports allow daisy-chaining to additional switches or direct connection to NexPro-NVR series recorders with full 1000Mbps throughput.

- Compact 312mm width fits standard 19-inch rack shelves or wall-mount enclosures in weak-current cabinets.

- Single-switch solution for up to 16 cameras simplifies procurement, spare parts management, and on-site troubleshooting.

© NexPro. Where Systems Connect.
Powered by Seetong Cloud & Seetong Client Compatible Infrastructure.
For bulk project pricing and switch-camera bundle quotations: contact your NexPro solutions team.