CCTV & SURVEILLANCE SYSTEM

NexPro 16CH 2-Bay 8MP PoE NVR (H.265+)

16 IP Channels | 16 PoE Ports | Dual HDD Bays | Seetong Cloud

PRODUCT IMAGES

16-Channel PoE NVR · Reference Rear Panel

Reference image shows a DC-type recorder inlet. Confirm the AC input/adapter arrangement and RCA audio connections for the final hardware configuration.

PRODUCT OVERVIEW

16-channel PoE network video recorder with 260Mbps incoming bandwidth. Supports IP camera input up to 8MP, with recording resolutions up to 5MP. Automatic camera discovery, addition and recording enable plug-and-play setup. Two SATA HDD bays support up to 14TB per HDD, with 2-channel playback and Seetong remote access.

| Product line | CCTV & Surveillance System |
| --- | --- |
| Series / family | PoE Network Video Recorder |
| Product variant | 16CH PoE NVR · 2 SATA HDD bays · Up to 8MP IP camera input |
| Applications | Small Warehouse, Restaurant, Retail |

MODEL IDENTIFICATION

| Product | Product / variant | Channels | Camera input | Storage |
| --- | --- | --- | --- | --- |
| NexPro PoE NVR | 16CH 2-Bay 8MP PoE NVR (H.265+) | 16 IP / 16 PoE | Up to 8MP | 2 SATA HDD bays Max. 14TB per HDD |

TECHNICAL SPECIFICATIONS

| Parameter | Specification |
| --- | --- |
| Network Video Input | 16 channels |
| Incoming Bandwidth | 260Mbps |
| Camera Input Resolution | Up to 8MP |
| Recording Resolutions | 5MP / 4MP / 3MP / 1080P / 960P / 720P |
| Video Compression | H.265+ / H.265 |
| Automatic Setup | Automatic camera discovery, addition and recording; plug-and-play setup without manual IPC addition |
| Simultaneous Playback | 2 channels |
| HDD Storage | 2× SATA interfaces; 2 HDD bays; up to 14TB per HDD |
| Video Outputs | 1× HDMI + 1× VGA; simultaneous output supported |
| Audio Output | 1× RCA |
| Two-way Audio Input | 1× RCA |
| LAN Interface | 1× RJ45, 10/100/1000Mbps auto-adaptive Ethernet port |
| PoE Interfaces | 16× 10/100Mbps auto-adaptive Ethernet ports; 48V standard PoE supply |
| USB Interfaces | 2× USB 2.0 |
| Network Protocols | IPv6, UPnP, SNMP, NTP, SADP, SMTP, PPPoE, DHCP |
| AC Supply | AC 220V; 100–240V wide-range input recommended (not confirmed) |
| Power Consumption | 25–30W, excluding HDDs and PoE load (reference) |
| Total PoE Power Budget | 150–200W reference range; model- and PSU-dependent; final rating to be confirmed |
| Operating Temperature | -10°C to +55°C |
| Weight | Approx. 5.0kg, excluding HDDs |
| Cloud and Remote Access | Seetong Cloud; remote live view and recorded-video playback via mobile app and PC client |

Power figures are reference values. The supplied 150–200W loaded-system estimate is not a verified full-load rating; confirm PSU sizing for the recorder, HDDs and maximum planned PoE load.

AI FUNCTIONS & ALARM LINKAGE

| Parameter | Specification |
| --- | --- |
| Human / Vehicle Detection | Accurate human and vehicle detection |
| Simultaneous Human Detection | Up to 3 people detected simultaneously |
| Perimeter Protection | Tripwire detection and region intrusion detection |
| NVR Alarm | Pop-up alarm on the NVR display |
| App Notifications | Alarm push notifications, including push when the app is inactive |
| Intelligent Playback | Human and vehicle event recording playback |

AI function compatibility should be confirmed with the selected IP cameras and firmware. App push notifications require an available network connection.

KEY FEATURES

1. Stable operation. 50% CPU headroom at full load, IP conflict adaptation and fault-tolerant recording with 99.99% completeness.*

2. Easy installation. Intuitive menus, IPC/router adaptation and automatic camera discovery, addition and recording for zero-configuration setup.

3. Efficient storage. H.265+ can save up to 90% of HDD space versus H.264; dual HDD bays and up to 14TB per HDD support extended recording.*

4. Fast remote access. Stated performance: 10% traffic and up to 5× smoother viewing, with rapid cloud access and remote playback.*

*Performance figures depend on test conditions and system configuration. Storage savings vary with scene and encoding settings; remote performance varies with the network.

PRODUCT FAMILY & ACCESSORY REFERENCE

| PoE NVR family · 4 / 8 / 16 ports This datasheet covers the 16CH model only. | Accessory reference · 8-port chassis shown Final 16CH accessories follow the project quotation. |
| --- | --- |

Accessory items shown include a power adapter, AC power cord, USB mouse, Ethernet cable and small accessory packs. Final accessory selection and quantities follow the project quotation.

APPLICATIONS & RFQ CONFIRMATION

| Typical applications | Confirm in the project RFQ |
| --- | --- |
| Small Warehouse Restaurant Retail | Confirm the 16CH PoE configuration, HDD capacity and AI camera compatibility. Confirm quantity, destination, packing, branding and documentation requirements. |

Project supply: MOQ — Confirm by project quantity · Lead time — Confirm by project list · FOB port — China main ports · OEM/ODM — Available; confirm project scope

PROJECT CONFIGURATION CHECKLIST

| Product configuration | 16 IP channels; 16 PoE ports; 2 SATA HDD bays; up to 14TB per HDD |
| --- | --- |
| Cameras and AI functions | Confirm IP camera models, input resolution, detection functions and alarm linkage. |
| HDD and recording plan | Confirm HDD capacity, recording mode, bitrate and required retention period. |
| Interfaces and accessories | Confirm AC input/adapter rating, RCA connections, recorder/HDD load, total PoE budget and accessories. |
| Finish, branding and packing | Confirm branding, packing and customization requirements for the order. |
| Quantity and destination | Confirm order quantity, delivery plan, destination, power plug and documentation. |

STORAGE RETENTION REFERENCE (H.265+)

Actual recording storage requirements vary with the surveillance scene. The figures below are for reference only and were calculated using H.265+ encoding. All values are recording days.

| Camera resolution | Recording channels | 2TB | 4TB | 6TB | 8TB | 14TB | 18TB* |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3MP | 2 | 114 | 228 | 341 | 455 | 796 | 1024 |
|  | 4 | 57 | 114 | 171 | 228 | 398 | 512 |
|  | 6 | 38 | 76 | 114 | 152 | 265 | 341 |
|  | 8 | 28 | 57 | 85 | 114 | 199 | 256 |
|  | 10 | 23 | 46 | 68 | 91 | 159 | 205 |
| 4MP | 2 | 102 | 205 | 307 | 410 | 717 | 922 |
|  | 4 | 51 | 102 | 154 | 205 | 358 | 461 |
|  | 6 | 34 | 68 | 102 | 137 | 239 | 307 |
|  | 8 | 26 | 51 | 77 | 102 | 179 | 230 |
|  | 10 | 20 | 41 | 61 | 82 | 143 | 184 |
| 5MP | 2 | 93 | 186 | 279 | 372 | 652 | 838 |
|  | 4 | 47 | 93 | 140 | 186 | 326 | 419 |
|  | 6 | 31 | 62 | 93 | 124 | 217 | 279 |
|  | 8 | 23 | 47 | 70 | 93 | 163 | 209 |
|  | 10 | 19 | 37 | 56 | 74 | 130 | 168 |

*The 18TB column is reproduced as a general storage reference. It does not confirm compatibility with an 18TB HDD.

HOW TO READ AND APPLY THE TABLE

Read a scenario. Select the camera resolution and recording-channel count, then read the storage-capacity column. The number is the reference duration in days.

Check project settings. The source does not specify bitrate, frame rate, recording schedule or usable-capacity allowance. Confirm these inputs when sizing storage for an installation.

Observe the source coverage. The reference covers 3MP, 4MP and 5MP at 2, 4, 6, 8 or 10 channels. No 8MP or 16-channel recording figures are provided.

APPLICATION TO THIS 16CH MODEL

| Recorder configuration | 16 IP channels; 2 SATA HDD bays; maximum 14TB per HDD |
| --- | --- |
| Available source rows | The supplied table covers 2, 4, 6, 8 and 10 recording channels. It does not include a 16-channel row. |
| Full 16-channel retention | Calculate separately using the planned recording settings and total usable HDD capacity; no 16-channel duration has been inferred here. |
| Capacity scope | The 18TB column is a general storage reference. The recorder’s specified limit remains 14TB per HDD, with two HDD bays. |

Source: supplied H.265+ storage reference table. Original numerical values have been preserved.